TEAC Z7000 Dragon Slayer upgrades

New Pitch control with push down lock 10K center detent. Alps

New Headphone pot with push down lock. Alps

New IEC power inlet connector with ground.

ALL 4558 and 4560 IC’s replaced by Burr Brown OPA2227PA 8 Mhz, 45mA per channel, no shutdown.

Add 0.1 uf 50V poly caps between pins 8 and 4 of each of the IC’s replaced

All 80 electrolytic capacitors are to be replaced with Nichicon low impedance type 105 deg. Non Polar. Polar and Power Supply.

Align to IEC standard test tapes.

Create and alignment tape with the Nakamichi for alignment and comparison purposes and touch up where necessary.

Calibrate to customer’s blank tapes as desired and calibrate the auto-cal.

The whole job start to finish $1500.00